SLIDE 1 — Title

Self-powered co-harvesting for batteryless sensing and monitoring in two-wheeled IoT applications

Giuseppe Pasquini, Maria Doglioni, Arianna Saviane, Davide Brunelli

University of Trento — IEEE STAR 2025

Notes:

Good morning, and thank you. Today I present our work on self-powered sensing for cycling applications. Our goal is to reduce dependence on small onboard batteries by harvesting energy directly from the bike, without friction or impact on riding performance.

SLIDE 2 — Cycling is becoming an IoT platform

• More sensors and wireless control

• Performance, safety, bike setup optimization

• Electronics are now part of modern bikes

Notes:

Modern bikes increasingly rely on electronics: telemetry, smart suspension, power meters, wireless shifting and connectivity. This improves performance and safety, but also creates a growing energy demand directly on the bike.

SLIDE 3 — The problem: too many batteries

• Up to 5–7 batteries on a high-end MTB

• Recharging burden and failures

• Environmental impact

→ We need autonomy, not more batteries

Notes:

Each device needs its own small battery. Riders must recharge and maintain many cells. This is inconvenient, costly, and not sustainable. Our challenge was: can we remove batteries from some of these systems entirely?

SLIDE 4 — Core idea (1 sentence)

Friction-free co-harvesting (TEG + PV) → battery-less bike sensing

Notes:

The concept is simple: use thermoelectric energy from braking and solar energy from ambient light, combine them with a smart power manager, and power sensors and communication without a battery.

SLIDE 5 — Hybrid system concept

• TEG → downhill bursts (braking heat)

• PV → continuous background energy

• Shared capacitor + smart power management

Notes:

The two energy sources are naturally complementary. Downhill: strong braking means short but high power bursts. Climbing or flat sections: little braking, but sunlight is always present. Together they create a stable energy profile.

SLIDE 6 — Thermoelectric harvesting (TEG)

• Installed on brake caliper

• Uses temperature gradient

• Produces mW-level bursts during braking

• No friction, no moving parts

Notes:

Brakes convert kinetic energy into heat. A TEG captures part of this wasted heat. The output is small but perfectly suitable for powering sensing and BLE communication bursts, especially during downhills.

SLIDE 7 — Solar harvesting (PV)

• Works in all lighting conditions

• µW to mW depending on sunlight

• Provides baseline power over the full ride

Notes:

The PV output is smoother and continuous. It keeps the system alive when the rider is not braking. This makes the hybrid approach more reliable in realistic outdoor scenarios.

SLIDE 8 — TEG on-field results

• Peak power: 2.3 mW (Ride 1), 3.2 mW (Ride 2)

• Harvested energy: up to 0.621 Wh

• Strong correlation: braking ↔ power

Notes:

In downhill tests, we measured milliwatt peaks and significant total energy. TEG alone can sustain continuous telemetry during descents, where fast data acquisition is most useful for performance analysis.

SLIDE 9 — PV results (typical daylight)

Indoor/shade: ~8–30 µW

Daylight: ~170 µW

Direct sunlight: ~1 mW

Notes:

Solar alone can support periodic sensing and keeps the system alive for the entire ride. Under strong sunlight, it reaches up to 1 mW, comparable to TEG bursts in light braking scenarios.

SLIDE 10 — Use Case #1: batteryless telemetry

• Brake temperature, IMU, suspension

• Continuous sampling during downhill (TEG)

• 0.77 s sampling period with PV only

Notes:

We achieved energy-neutral operation: continuous downhill monitoring and sub-second periodic sampling in daylight using only harvested energy. No battery is required for this telemetry application.

SLIDE 11 — Use Case #2: powering commercial devices

• Works today: remotes, shifters, power meters (1–4 mW)

• Not yet: droppers, derailleurs, active suspension (30–300 mW)

Notes:

We can already power low-power commercial components continuously. Actuation systems are still too energy-hungry, but improvements in both harvesting and consumption will gradually close the gap.

SLIDE 12 — IoT pipeline

BLE → Smartphone → Cloud (MQTT) → Dashboard

Real-time data and visualization

Notes:

To validate our system, we built a complete IoT pipeline for real-time visualization and data storage, without adding extra power requirements on the bike.

SLIDE 13 — Key takeaways

• Hybrid harvesting = reliable power profile

• Telemetry → batteryless today

• Commercial devices → partially supported

• Zero friction, zero maintenance

Notes:

The architecture works in real outdoor conditions. It is practical, lightweight, and already eliminates the need for batteries in several scenarios while reducing maintenance.

SLIDE 14 — Conclusions & future work

• Improve heat transfer and efficiency

• Expand to e-bikes and more sensors

• Towards fully batteryless bike electronics

Notes:

This is a first step. With better TEG coupling and lower-power electronics, we can move toward fully batteryless bikes in the near future. Thank you.